ECE3003 – Microcontroller And Its Applications

TASK - IV

Name : Guneet Arora

Registration Number:  16BEC0578

Slot : L37 + L38

Submitted to : Prof. Chitra P

Program 1

Code

ORG 0000H

XX: MOV DPTR,#MYDATA

MOV TMOD,#20H

MOV TH1,#-3

MOV SCON,#50H

SETB TR1

MOV R1,#14

AGAIN: CLR A

MOVC A,@A+DPTR

MOV SBUF,A

HERE:  JNB TI,HERE

CLR TI

INC DPTR

DJNZ R1,AGAIN

SJMP XX

MYDATA: DB 'VIT UNIVERSITY'

END

Output

Program 2

Code

ORG 0000H

MOV TMOD,#20H

MOV TH1,#-6

MOV SCON,#50H

SETB TR1

HERE:JNB RI,HERE

MOV A,SBUF

MOV P1,A

CLR RI

SJMP HERE

END

Output

‘C’ from UART to P1.

‘A’ from UART to P1.

Task 4A

ORG 300H

DB "India is our country."

ORG 000H

MOV R0, #20H

SETB P1.2

MOV TMOD, #20H

MOV SCON, #50H

MOV C, P1.2

JC L1

MOV TH1, #-6

SJMP L2

L1 : MOV TH1, #-3

L2: MOV DPTR, #300H

SETB TR1

BACK : CLR A

MOVC A, @A+DPTR

MOV SBUF, A

HERE:  JNB TI,HERE

CLR TI

INC DPTR

DJNZ R0, BACK

END

Output

Task 4B

Code

ORG 300H

DB "GUNEET ARORA"

ORG 200H

DB "16BEC0578"

ORG 000H

MOV C, P1.2

JC L1

MOV A, PCON

SETB ACC.7

MOV PCON, A

MOV TH1, #-6

MOV DPTR, #200H

SETB TR1

BACK: CLR A

MOVC A, @A+DPTR

MOV SBUF, A

HERE: JNB TI, HERE

CLR TI

INC DPTR

DJNZ R1, BACK

L1: MOV TH1, #-3

MOV DPTR, #300H

SETB TR1

BACK2: CLR A

MOVC A, @A+DPTR

MOV SBUF, A

HERE2: JNB TI, HERE2

CLR TI

INC DPTR

DJNZ RO, BACK2

SJMP START

END

Output

Name : Guneet Arora

Registration Number : 16BEC0578

Program 1

Code

ORG 000H

LJMP MAIN

ORG 50H

MAIN:  MOV P0,#0FFH

MOV P1,#00H

MOV IE,#82H

MOV TMOD,#02H

MOV TH0,#02H

SETB TR0

BACK:MOV A,P0

MOV P1,A

SJMP BACK

ORG 0BH

CPL P2.1

RETI

END

Output

Program 2

Code

ORG 000H

LJMP MAIN

ORG 50H

MAIN:

MOV P2,#00H

MOV P1,#00H

SETB P1.2

CLR P1.7

MOV TMOD,#20H

MOV TH1,#-3

MOV SCON,#50H

MOV IE,#90H

SETB TR1

BACK:MOV C,P1.2

MOV P1.7,C

SJMP BACK

ORG 23H

JNB TI,L1

CLR TI

L1:JNB RI,L2

MOV A,SBUF

MOV P2,A

CLR RI

L2:RETI

END

Output

Task 4C

Code

ORG 00H

LJMP MAIN

ORG 023H

JNB RI,L2

MOV A,SBUF

MOV P0,A

CLR RI

L2:RETI

ORG 00BH

CPL P1.0

RETI

ORG 050H

MAIN:MOV P0,#0FFH

MOV IE,#92H

MOV SCON,#50H

MOV TMOD,#22H

MOV TH1,#-6

MOV SCON,#50H

SETB TR1

MOV TH0,#0A3H

SETB TR0

SJMP  $

END

Output

Letter ‘A’

Letter ‘B’

Analyzer Window Output

Task 4D

Write a 8051 assembly program using timer 0 to generate a 500 Hz and timer 1 to 7kHz square wave frequency on P1.0 and P1.1 respectively using Interrupts. Then examine the frequency using the KEIL IDE inbuilt Logic Analyzer.

Screenshot:

Code:

ORG 0000H

LJMP MAIN

ORG 000BH

DJNZ R0,START

CPL P1.0

MOV R0,#5

MOV TH0,#46H

START:RETI

ORG 001BH

CPL P1.1

RETI

ORG 0030H

MAIN:

MOV TMOD,#22H

MOV R0,#5

MOV TH0,#46H

MOV TH1,#0BEH

MOV IE,#10001010B

SETB TR0

SETB TR1

BACK: SJMP BACK

END